Agenda Item: 2/03

= application site

| 77 Hillview Road, Pinner, HA5 4PB | P/3564/22 |
| --- | --- |

LOCATION PLAN

LONDON BOROUGH OF HARROW

PLANNING COMMITTEE

18th January 2023

| APPLICATION NUMBER: | P/3564/22 |
| --- | --- |
| VALIDATE DATE: | 18/10/2022 |
| LOCATION: | 77 HILLVIEW ROAD |
| WARD: | HATCH END |
| POSTCODE: | HA5 4PB |
| APPLICANT: | MR & MRS MANGA |
| AGENT: | TEN POINT FIVE ARCHITECTURE |
| CASE OFFICER: | CATRIONA COOKE |
| EXPIRY DATE: | 07/12/22 |

PROPOSAL

Variation of condition 2 (approved plans) attached to planning permission P/3331/16 allowed on appeal reference APP/M5450/W/16/3161002 to allow alterations to elevations

RECOMMENDATION

The Planning Committee is asked to:

- Agree the reasons for approval as set out in this report, and

- Grant planning permission subject to the conditions listed in Appendix 1 of this report:

REASON FOR THE RECOMMENDATIONS

The variations to the approved development are considered to be modest and have an acceptable impact on the character and appearance of the area.

Accordingly, weighing up the development plan policies and proposals along with other material considerations including comments received in response to notification and consultation as set out below, Officers consider and conclude that, subject to planning conditions, the proposed development is acceptable and worthy of support. In accordance with the National Planning Policy Framework, including its presumption in favour of sustainable development, and subject to conditions, officers recommend that the application is approved.

INFORMATION

This application is reported to Planning Committee at the request of a nominated member in the public interest and therefore falls within proviso A of the Scheme of Delegation.

This application was deferred from the December planning committee to permit consideration of this alongside the concurrent planning application P/3574/22.

| Statutory Return Type: | E13 Minor Dwellings |
| --- | --- |
| Council Interest: Net additional Floorspace: | N/A 52 sqm |
| GLA Community Infrastructure Levy (CIL) Contribution (provisional): | N/A |
| Local CIL requirement: | N/A |

HUMAN RIGHTS ACT

The provisions of the Human Rights Act 1998 have been taken into account in the processing of the application and the preparation of this report.

EQUALITIES

In determining this planning application, the Council has regard to its equalities obligations including its obligations under section 149 of the Equality Act 2010.

For the purposes of this application there are no adverse equalities issues.

S17 CRIME & DISORDER ACT

Policy D11 of the London Plan (2021) and Policy DM1 of the Development Management Polices Local Plan require all new developments to have regard to safety and the measures to reduce crime in the design of development proposal. It is considered that the development does not adversely affect crime risk. However, a condition was not imposed on the appeal scheme and therefore it would be unreasonable for a condition to be attached.

- SITE DESCRIPTION

  - The site contains a recently completed pair of semi-detached properties located on the south eastern side of Hillview Road.

  - The site has a maximum width of 15.20m and a maximum depth of 48.8m. The footprint of the site is 721.52sq m.

  - The site is set within a row of residential dwellinghouses, with semi-detached houses interspersed with detached houses to the north and to the south. The neighbour immediately to the south is No. 79 Hillview Road, with No. 75 Hillview Road to the north.   The character of the area is varied.

  - The site is located in a Critical Drainage Area and has no other statutory   designations.

- PROPOSAL

2.1     	The application seeks to vary condition 2 of appeal Decision APP/M5450/W/16/3161002 to allow the following amendments:

- V1 - Reduction of Brick panel on the front elevation to one brick width

- V2 - Centralised bay window/first floor windows centralised to bay window

- V3 - Reduction in width of front gable to match the width of the first-floor windows.

- V4 - Removal of pitched roof over bay windows

- V5 - Replacement of brick on edge parapet wall at ground floor with an aluminium coping.

- V6 - Render above front doors has been lowered to match the render from the front and side elevations.

- V7 - Removal of false Chimneys

- V8- Replacement of triangular coping over party wall with brick on edge detailing.

- V9 – Replacement of white UPVC windows with Black UPVC windows

- V10 - Reduction of brick panel on the rear elevation to one brick width

- V11 - Angled brickwork above rear porch (77A).

- V.12 - Change from patio doors to bi-fold doors at rear

- V.13 – One large rooflight changed to two small rooflights.

- V.14 – Increase in height of first floor side windows (by lowering the cil)

- V.15 – Increase in height of ground floor side windows (by lowering the cil) – consent scheme showed clear glazing obscure glazing has been installed.

- V.16 – New obscure window at ground floor.

- RELEVANT PLANNING HISTORY

  - A summary of the relevant planning application history is set out in the table below:

| Ref no. | Description | Status & date of decision |
| --- | --- | --- |
| P/3331/16 | Re-development to provide a pair of semi-detached two storey dwellings with habitable roofspace; new vehicular access parking bin / cycle storage landscaping and boundary treatments | Refused 03/10/2016 Appeal Allowed 10/02/2017 |
| P/1982/17 | Details pursuant to Conditions 3 (Materials) and 4 (Boundary Treatment) attached to planning permission P/3331/16 allowed on appeal reference APP/M5450MW/16/3161002 dated 10/2/17 for Re-development to provide a pair of semi-detached two storey dwellings with habitable roofspace; new vehicular access parking bin / cycle storage landscaping and boundary treatments | Approved 07/07/2017 |
| P/1391/20 | CERTIFICATE OF LAWFUL DEVELOPMENT (PROPOSED): Alterations to roof to form end gable, rear dormer and insertion of three rooflights on front roofslope; Front porch; Single storey side extensions to both sides; Two storey rear extension | Refused 15/06/2020 |
| Reasons for refusal: - The cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than 50 cubic meters. The proposal would therefore not be within the tolerances of Schedule 2, Part 1 Class B.1(d)(ii) of The Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). - The proposal is NOT therefore a lawful development. |  |  |
| P/0368/20 | Lawful implementation of planning permission P/3331/16 (construction of pair of semi-detached two storey dwellings with habitable roofspace (2 x 4 beds); new vehicular access parking bin / cycle storage landscaping and boundary treatments) allowed on appeal APP/M5450/W/16/3161002 | Granted 20/04/2020 |
| P/3574/22 | Redevelopment to provide two X two storey (4 bed) semi-detached dwelling houses with habitable roofspaces; proposed vehicle access; separate amenity space; bin and cycle stores | Under assessment |

- CONSULTATION

4.1	A total of 41 consultation letters were sent to neighbouring properties regarding this application. A total of 9 responses were received.

4.2	A site notice was posted on 14 November 2022 expiring on 5th December 2022.

4.3       A summary of the consultation responses received along with the Officer comments are set out in the table below: -

- A total of 16 violations should not be allowed as will set a precedent for other developers. Officer comment: Section 73 of the Planning Act allows a developer to submit an application for retrospective planning permission without complying with the original permission, as is the case under this application. The LPA is required to determine such application as it would for a new application in accordance with the relevant development in place at the time the application has been made. - Materials are out of keeping with surrounding area Officer comment: See 6.3. below - Front gables inconsistent with neighbouring properties. Officer comment: See 6.3.6 below - Window panes of bay window inconsistent with first floor window Officer comment: See 6.3.5 below - The brick panel reduction in front elevation is an improvement but the two vertical full height drainpipes have become an unsightly feature. Officer comment: See 6.3.4 below - The cycle storage, waste bin storage, box tree hedging separating the grass at the front and low level brick wall along the boundary with pavement have not yet been completed. Officer comment: See 6.3.14 below

Consultee and Summary of Comments

Hatch End Association - The Hatch End Association is very concerned that developers are willing to ignore the planning regime when it suits them, and approval of this application is likely to encourage others to follow suit. The Hatch End Association seeks to preserve the essential character of Hatch End and sees no reason that blatant ignoring of the rules should be allowed. Landscape Officer - No comments received

- POLICIES

5.1	Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that:

‘If regard is to be had to the Development Plan for the purpose of any determination to be made under the Planning Acts, the determination must be made in accordance with the Plan unless material considerations indicate otherwise.’

5.2	The Government has issued the National Planning Policy Framework [NPPF 2021] sets out the Government’s planning policies for England and how these should be applied and is a material consideration in the determination of this application.

5.3	In this instance, the Development Plan comprises The London Plan 2021 [LP] and the Local Development Framework [LDF]. The LDF comprises The Harrow Core Strategy 2012 [CS], Harrow and Wealdstone Area Action Plan 2013 [AAP], the Development Management Policies Local Plan 2013 [DMP], the Site Allocations Local Plan [SALP] 2013 and Harrow Local Area Map 2013 [LAP].

5.4	A full list of all the policies used in the consideration of this application is provided as Informative 1.

6.0	ASSESSMENT

6.1	The main issues are:

  - Principle of the Development

  - Design, Character and Appearance of the Area

  - Residential Amenity

  - Traffic and Parking

  - Waste and Servicing

  - Flood Risk and Drainage

  - Biodiversity

  - Fire Safety

6.2	Principle of Development

The relevant policies are:

  - The National Planning Policy Framework (2021)

  - The London Plan (2021): H1, H2, H9

  - Harrow Development Management Policies (2013): DM24

  - Harrow's Core Strategy (2012): CS1

  - Supplementary Planning Document – Garden Land Development (2013)

6.2.1 	The principle of the development has been established in the appeal allowed under reference APP/M5450/W/16/3161002.  As such, matters relating to the principle of the development, the size of accommodation proposed, transport impacts, landscaping and fire safety do not need to be considered under this application again as these elements would remain unchanged from the original approval.

6.3       	Design, Character and Appearance of the Area

The relevant policies are:

  - National Planning Policy Framework (2021)

  - The London Plan 2021: D3

  - Harrow Core Strategy 2012: CS1

  - Harrow Development Management Polices Local Plan (2013): DM1, DM22, DM23

  - Residential Design Guide (2010)

  - The London Plan Housing Supplementary Planning Guidance (2016)

  - Code of Practice for Storage and Collection of Refuse and Materials for Recycling in Domestic Properties (2016)

6.3.1    	Policy D3.D(1) of the London Plan states that development should in terms of form and layout, enhance local context by delivering buildings and spaces that positively respond to local distinctiveness through their layout, orientation, scale, appearance and shape, with due regard to existing and emerging street hierarchy, building types, forms and proportions. Policy D3.D(11) goes on to states that in terms of quality and character, developments should respond to the existing character of a place by identifying the special and valued features and characteristics that are unique to the locality and respect, enhance and utilise the heritage assets and architectural features that contribute towards the local character.

6.3.2 	Policy DM1 of the DMP gives advice that ‘’all development proposals must achieve a high standard of design and layout. Proposals which fail to achieve a high standard of design and layout, or which are detrimental to local character and appearance, will be resisted”.

6.3.3 	As noted above the development has been largely completed on site.  The development is to the same footprint and scale as the approved plans.  In allowing the appeal the Inspector did not include a condition restricting permitted development rights for the dwellings.  This application seeks to retrospectively vary the approved plans which are assessed below.

6.3.4 	V1 - Reduction of Brick panel on the front elevation to one brick width

It is considered that this element of the development by reducing the 1.8m wide brick panel as approved to a 0.2m wide panel is an improvement to the approved plans creating a more visually cohesive design.  It is noted that objections have been received relating to the visually prominent down pipes.  However, these are normal features of properties in the area.  Therefore, it is considered that this element has an acceptable impact on the character of the area.

6.3.5	V2 - Centralised bay window/first floor windows centralised to bay window

This amendment is considered to be minor and in centralising the bay windows and first floor windows the development has a balanced frontage which is in keeping with the area.

6.3.6	V3 - Reduction in width of front gable to match the width of the first-floor windows.

As noted by the Inspector in allowing the appeal, the area is mixed in character and appearance.  While reduction in width of the front gables is a significant variation to the approved plans, it is considered that given the varied character of the area this element does not have a negative impact on the character of the area.

6.3.7	V4 - Removal of pitched roof over bay windows

There are examples within the area of single storey front extension with flat roofs.  It is considered that this is a minor change which does not have a harmful impact on the character of the area.  Furthermore, as noted above the Appeal Decision does not remove permitted development right and it is considered that the pitched roofs could be removed under Class A of the General Permitted Development Order.  However, should member consider that the pitched roofs are required the applicant has confirmed that these could be added and could be secured by condition.

6.3.8	V5 - Replacement of brick on edge parapet wall at ground floor with an aluminium coping.

This is considered to be a minor amendment and is not clearly visible within the streetscene.  Therefore, it is considered that this variation has negligible impact on the streetscene.

6.3.9	V6 - Render above front doors has been lowered to match the render from the front and side elevations.

This element is set back from the main frontage of the building and is considered to be a minor alteration which has a negligible impact on the character of the area.

6.3.10	V7 - Removal of false Chimneys

As noted above the Inspector did not impose a condition removing permitted development rights.  Chimney can be removed without requiring planning permission.   However, should Members consider that the chimneys should be reinstated the applicant has indicated that these could be provided which could be secured by a condition.

    - V8- Replacement of triangular coping over party wall with brick on edge detailing.

It is considered that the brick on edge detailing is acceptable and does not impact on the character of the dwellings or the streetscene.

    - V10 - Reduction of brick panel on the rear elevation to one brick width

V11 - Angled brickwork above rear porch (77A).

These elements are to the rear of the property and are considered to be minor alterations which do not impact of the character of the dwelling or the surrounding area.

    - V9 – Replacement of white UPVC windows with Black UPVC windows

V.12 - Change from patio doors to bi-fold doors at rear

V.13 – One large rooflight changed to two small rooflights.

V.14 – Increase in height of first floor side windows (by lowering the cil)

V.15 – Increase in height of ground floor side windows (by lowering the cil) – consented scheme showed clear glazing obscure glazing has been installed.

V.16 – New obscure window at ground floor.

These elements could be carried out under permitted development.  While it is noted that the replacement of the white upvc windows with black frames is contrary to the details approved under P/1982/17.  However, had white upvc windows been installed the future occupier could have changed these at a future date.

    - In addition to the variations listed above the roof tiles of the dwellings are not as approved under P/3331/16.  However as noted above given that permitted development rights have not been withdrawn from these dwellings, the roof tiles could be replaced under permitted development.  Also, it is noted that the boundary treatment of the front garden and the cycle/bins stores have not been constructed.  These elements were pre-occupation conditions, given that the dwellings are not yet occupied this condition has not been breached.  A condition is recommended to ensure that these are completed prior to occupation.

    - In summary, the variations are not considered to result in significant harm to the host dwellings or the surrounding area to warrant refusal.

6.4       Residential Amenity

The relevant policies are:

  - Harrow Core Strategy 2012:CS1

  - Harrow Development Management Polices Local Plan (2013): DM1, DM2, DM27

  - London Plan Policy (2021): D3, D6

  - Residential Design Guide (2010)

  - The London Plan Housing Supplementary Planning Guidance (2016)

Impacts on neighbouring properties

6.4.1    	Policy DM1 of the DMP seeks to ensure that “proposals that would be detrimental to the privacy and amenity of neighbouring occupiers, or that would fail to achieve satisfactory privacy and amenity for future occupiers of the development, will be resisted”.

6.4.2   	As noted above the development is to the same footprint and scale as the approved scheme and therefore it is considered that there would be no additional harm to the neighbouring residents.  While additional windows have been added to the flank elevations these are all obscure glazed and are considered to have an acceptable impact on the amenities of neighbouring occupiers.

Accessible Homes

6.4.3	Policy DM2 of the DMP seeks to ensure that all new housing is built to ‘Lifetime Homes’ standards.

6.4.4	Policy CS1.K of the Harrow Core Strategy requires all new dwellings to comply with the requirements of Lifetime Homes.

6.4.5	While the above policies require compliance with Lifetime Home Standards, in October 2015 these standards were replaced by New National Standards which require 90% of homes to meet Building regulation M4 (2) - ‘accessible and adaptable dwellings’.

6.4.6	The Inspector attached a condition to the approval of the appeal to ensure that the development is constructed to the specification of Part M, M4 (2).  This condition has been recommended.

Secure by Design

6.4.7	The Inspector did not attach a condition to the approval and therefore it would be unreasonable to attach a condition to this application.

6.5       Traffic, Parking and Servicing

The relevant policies are:

- Harrow Core Strategy 2012:CS1

- Harrow Development Management Polices Local Plan (2013): DM42. DM43

- London Plan (2021): T4, T5, T6

6.5.1	The Inspector raised no concerns regarding transport and parking.  A condition was imposed on the appeal approval to ensure that offstreet parking be laid out and made available for use prior to occupation.  This condition has been recommended.

7.0          CONCLUSION AND REASONS FOR APPROVAL

  - While it is regrettable that the variations to the consented scheme were not sought prior to completion of development.  The completed development has an acceptable impact on the street scene.

  - For all the reasons considered above, and weighing up the development plan policies and proposals, and other material considerations including comments received in response to notification and consultation as set out above, this application is recommended for grant.

Checked

| Mehdi Rezaie Head of Development Management (Interim) 22nd December 2022 | Viv Evans Chief Planning Officer 22nd December 2022 |
| --- | --- |

APPENDIX 1: Conditions and Informatives

Conditions - Approved Plans and Documents The development hereby permitted shall be retained in accordance with the following documents and plans: C243-01 Rev A, C243-04 Rev A, C243-05 Rev A, C243-06 Rev A, C243-09 Rev A, C319-51 Rev A, C319-52 Rev A, C319-53 Rev A, C319-55 Rev A, C319-56 Rev A, C319-57 Rev A, C243-33 Rev A, C243-34 Rev A REASON: For the avoidance of doubt and in the interests of proper planning. - Boundary Treatment The approved boundary treatment between the front gardens of the houses shall be implemented on site prior to first occupation. REASON: To safeguard the appearance and character of the area, and to enhance the appearance of the development in accordance with Policy DM1 of the Harrow Development Management Local Plans Policy (2013). - Refuse store Other than on collection days, the refuse/waste bins shall at all times be stored in the approved refuse/waste storage area on plan C319-52 Rev A. The refuse/waste storage area shall be retained and kept available for its intended purpose at all times thereafter REASON: To safeguard the appearance and character of the area, and to enhance the appearance of the development in accordance with Policy DM1 of the Harrow Development Management Local Plans Policy (2013). - Parking All off-street parking shall be laid out and made available for use in accordance with plan C319-52 Rev A prior to the first occupation of the approved houses and shall be retained and kept available for their intended purpose at all times thereafter. - Accessible Units. The development hereby permitted shall be constructed to the specifications of: "Part M, M4(2), Category 2: Accessible and Adaptable Dwellings" of the Building Regulations 2013 and thereafter retained in that form. REASON: To ensure that the development is capable of meeting ‘Accessible and Adaptable Dwellings’ standards in accordance with Policy D7 of The London Plan 2021, policy CS1.K of The Harrow Core Strategy 2012 and policies DM1 and DM2 of the Development Management Policies Local Plan 2013. INFORMATIVES: - Policies The following policies are relevant to this decision: The National Planning Policy Framework (2021) London Plan 2021: D3, D12, T5, T6.1 The Harrow Core Strategy 2012: CS1. Harrow Development Management Policies Local Plan 2013: DM1, DM2, DM42, DM45 Relevant Supplementary Documents: Supplementary Planning Document: Sustainable Building Design (2010) Supplementary Planning Document: Garden Land Development (2013) The London Plan Housing Supplementary Planning Guidance (2016) Code of Practice for Storage and Collection of Refuse and Materials for Recycling in Domestic Properties (2016) - Street numbering Harrow Council is responsible for the naming and numbering of new or existing streets and buildings within the borough boundaries. The council carries out these functions under the London Government Act 1963 and the London Building Acts (Amendment) Act 1939. All new developments, sub division of existing properties or changes to street names or numbers will require an application for official Street Naming and Numbering (SNN). If you do not have your development officially named/numbered, then then it will not be officially registered and new owners etc. will have difficulty registering with utility companies etc. You can apply for SNN by contacting technicalservices@harrow.gov.uk or on the following link. http://www.harrow.gov.uk/info/100011/transport_and_streets/1579/street_naming_and_numbering

APPENDIX 2: SITE PLAN

APPENDIX 3: PLANS

Approved Site plan

Existing Site plan

Approved Elevations

Existing Elevations

APPENDIX 4: SITE PHOTOS

APPENDIX 5: APPEAL DECISION

This page has been left intentionally blank